Understanding and using a compass

- Compass orientation

  - Cardinal points

    - North

    - South

    - East

    - West

  - Ordinal directions

  - Azimuth or bearing

- Identifying compass parts

  - Base plate

    - Holds other parts together

    - Provides a straight edge

    - Provides a measuring device

  - Direction of travel arrow

  - Magnetic needle

    - Points to magnetic north

    - Orients the compass to north

  - Compass housing

    - Turntable

    - Degree graduations

  - Orienting lines

- Facing an azimuth (bearing)

  - What is an azimuth

    - Direction relative to magnetic north

    - North = 0 degrees

    - 360 degrees clockwise

  - Steps in facing a bearing

    - Hold compass properly

      - Flat and level

      - Away from metal

    - Rotate compass dial placing azimuth on

- Direction of travel arrow

    - Rotate body to align N arrow with N on

- Housing

    - Sight a landmark along direction of travel

    - Walk to the landmark

- Shooting an azimuth

  - Direction to a landmark

  - Pick out a landmark

  - Hold compass flat and level

  - Aim direction of travel arrow at landmark

  - Rotate compass housing

    - Keep DOT arrow aimed at landmark

    - Magnetic north aligned with orienting

- Arrow

  - Read azimuth from compass housing